Release Notes

## SPAR Release V 321 – Patch 1-8ZCXB

| Xerox Product: | Xerox DCCS 50 |
| --- | --- |
| Document ID: | dc03rn0733 |
| Date: | 15-May-03 |
| Part Number: | N/A |

# Software Release Contents

This release consists of the following components:

| DCCS 50 Controller Software | Version 321 – Patch 1-8ZCXB |
| --- | --- |

## Purpose

This release updates the system for the following items:

- DC randomly stops accepting jobs from the network computers which cause a job-queue lockup (714130506)

- The Print Save/Delete feature does not work reliably. Occasionally the Print Save feature will not retain a copy of the job in the Completed Job Queue.

NOTE: This patch does not contain fixes from the previous patches. If you have any previous patches already loaded, all should be listed on the configuration sheet. The previous patches are NOT required to load this new patch.

## Installation Instructions

- Print a Configuration Report.

- Save the 1-8ZCXB.zip file to your hard drive, then extract 1-8ZCXB.ps file to your harddrive.

- Open the web page of the DCCS50 Golden Gate controller (Product Code – MFY) by typing in the IP address of the printer within your Web browser.

- Select Print. Select Browse and navigate to the location of the file, 1-8ZCXB.ps and select Open. Select the Print button to submit the patch to the controller.

- The printer will automatically reboot and print a configuration page.

- Compare the Configuration Report to the original one from Step 1 to make sure you retain all settings. If they are incorrect make the appropriate changes, at this time a second reboot may be required. Also, verify that the patch is installed by noting that 1-8ZCXB is shown under Machine Information (IDFE Controller Software Version).

New Configuration Report will appear as follows:

Machine Information

IDFE Controller Software Version	321; 7/10/2002

(3.14: 3/7/2002)

1-8ZCXB

# SPARs Fixed

This release addresses the following SPAR's:

| SPAR # | OPCO | Eureka # | Description |
| --- | --- | --- | --- |
| 7141305061 | XCL | 169967 | - DC will no longer randomly stop accepting jobs from the network computers |

1 indicates this is a DCTSO Known Solution, therefore a SPAR is not required to dispatch this release for the fixes listed above.

# Problems Fixed since v321 Launch

NOTE: This patch does not contain fixes from the previous patches. If you have any previous patches already loaded, all should be listed on the configuration sheet. The previous patches are NOT required to load this new patch.

Additional problems fixed since Maintenance Release V 321

| SPAR # | Version | Eureka # | Description |
| --- | --- | --- | --- |
| 9310937031 | 1-94KF9 | 169521 | The user can now delete a scan template that contains a space within the name |
| 1716718311 | 1-7NJXZ | 162790 | The user can now select and print a quantity greater then 99 via the Web UI Print option. |
| N/A | 1-7NJXZ | N/A | After printing 16,384 jobs all subsequent pages will no longer be blank. |
| N/A | 1-7NJXZ | N/A | ColorCal will no longer fail intermittently with “Target not Found” error messages. |

1 indicates this is a DCTSO Known Solution, therefore a SPAR is not required to dispatch this release for the fixes listed above.

# Dependencies

- This particular software is for the Golden Gate (Product Code – MFY) controller, V 321 SW must already be installed. Controller Software should be used in conjunction with IOT version 11.50.3 and the CentreWare Utilities and Print Drivers contained in the DCCS50 CentreWare release Version 3.2.