Release Notes

| Program: AV610 | Type of Change: Driver (Hardware, firmware, driver, XPA. Etc) Release Date: 01/16/2007 | Revision Number: 1.30.0116’07 Replaced Revision: 1.30.0720’06 Creation Date: 01/16/2007 |  |
| --- | --- | --- | --- |
| Author or Designer: Tortoise |  |  |  |
| What Changed? | 1. modify the low level driver, fixed calibration problem. |  |  |
| What has been Fixed? | - 修正 scan color mode, 影像偏紅的問題. |  |  |
| Known Defects |  |  |  |
| Required Files And Revision Level | File Name |  | Revision |
|  | AV610.dll |  | 0,0,20,0 |
| Details: |  |  |  |

Test Overview:

Scanner Driver Info & Test Setup

Enter information about the scanner/driver test.

| Scanner Model Name: | AV610 | I/O driver version: | 0,0,20,0 |
| --- | --- | --- | --- |
| Firmware Version: | 1.06 | Application: | Photoshop 7.0 |
| TWAIN driver version: | 1.30.0116’07 |  |  |

| # Installer Testing |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Step | Win98 | Win2K | WinXP | WinVista | Comments |
| - Install |  |  |  |  |  |
| - Model name information | OK |  | OK |  |  |
| - Scan(200dpi A4 B/W) | OK |  | OK |  |  |
| - Uninstall |  |  |  |  |  |
| - Model name information | OK |  | OK |  |  |
| - Driver remove | OK |  | OK |  |  |
| - Scan Test |  |  |  |  |  |
| - Install driver then reboot PC | OK |  | OK |  |  |
| - Scan(200dpi A4 B/W) | OK |  | OK |  |  |

| # Basic Functionality Testing |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Step | Win98 | Win2K | WinXP | WinVista | Comments |
| - Mode Test |  |  |  |  |  |
| - Color |  |  | OK |  |  |
| - Gray |  |  | OK |  |  |
| - B/W |  |  | OK |  |  |
| - Brightness Test |  |  |  |  |  |
| - -50 |  |  | OK |  |  |
| - 0 |  |  | OK |  |  |
| - 50 |  |  | OK |  |  |
| - Contrast Test |  |  |  |  |  |
| - -50 |  |  | OK |  |  |
| - 0 |  |  | OK |  |  |
| - 50 |  |  | OK |  |  |
| - Resolution Test |  |  |  |  |  |
| - 100 |  |  | OK |  |  |
| - 150 |  |  | OK |  |  |
| - 200 |  |  | OK |  |  |
| - Simplex\Duplex |  |  |  |  |  |
| - Simplex |  |  | OK |  |  |
| - Duplex |  |  | N/A |  |  |
| - Simulate Mode Test |  |  |  |  |  |
| - A4, 200, B/W |  |  | OK |  |  |
| - A4, 100, Gray |  |  | OK |  |  |
| - A4, 150, Color |  |  | OK |  |  |
| - A4, 200, Gray |  |  | OK |  |  |
| - A4, 150, B/W |  |  | OK |  |  |
| - A4, 200, Color |  |  | OK |  |  |
| - A4, 100, B/W |  |  | OK |  |  |
| - A4, 150, Gray |  |  | OK |  |  |
| - A4, 100, Color |  |  | OK |  |  |
| - Button Test |  |  |  |  |  |
| - Scan button |  |  | OK |  |  |
| - Cancel button |  |  | N/A |  |  |
| - Duplex button |  |  | N/A |  |  |
| - Simplex button |  |  | N/A |  |  |
| - Email button |  |  |  |  |  |
| - Copy button |  |  |  |  |  |

| # Special Function Testing |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Step | Win98 | Win2K | WinXP | WinVista | Comments |
| - Hardware Image Processing |  |  |  |  |  |
| - Color drop - None |  |  | OK |  |  |
| - Color drop - R |  |  | OK |  |  |
| - Color drop - G |  |  | OK |  |  |
| - Color drop -B |  |  | OK |  |  |
| - Software Image Processing |  |  |  |  |  |
| - Deskew + Crop (Color) |  |  | N/A |  |  |
| - Crop (Gray) |  |  | N/A |  |  |
| - Deskew + Crop (B/W) |  |  | N/A |  |  |
| - Dynamic Threshold |  |  | N/A |  |  |
| - High Quality (Gray) |  |  | N/A |  |  |
| - Invert (B/W) |  |  | OK |  |  |
| - Blank Page Detection (Color, Gray, B/W) |  |  | N/A |  |  |
| - Rotation (90 Degrees CW) – B/W |  |  | N/A |  |  |
| - Rotation (180 Degrees) – Gray |  |  | N/A |  |  |
| - Rotation (90 Degrees CCW) – Color |  |  | N/A |  |  |
| - Color drop - None |  |  | N/A |  |  |
| - Color drop - R |  |  | N/A |  |  |
| - Color drop - G |  |  | N/A |  |  |
| - Color drop -B |  |  | N/A |  |  |
| - Multi-feed Detection |  |  | N/A |  |  |
| - OverScan |  |  | N/A |  |  |
| - Power Saving |  |  | OK |  |  |
| - Mirror |  |  | OK |  |  |
| - Preview (Color, Gray, B/W) |  |  | OK |  |  |
| - Flip Side Rotation (Fanfold) |  |  | N/A |  |  |
| - Edge Fill |  |  | N/A |  |  |
| - Background Setting (Crop) |  |  | N/A |  |  |

| # Adverse Conditions Testing |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Step | Win98 | Win2K | WinXP | WinVista | Comments |
| - Jamming Test |  |  | OK |  |  |
| - Jam-Release Test |  |  | OK |  |  |
| - Power Off Test |  |  | OK |  |  |
| - Cancel Scan Test |  |  | OK |  |  |
| - ADF Cover Open |  |  | N/A |  |  |
| - Cable disconnect Test |  |  | OK |  |  |

| Scanner Driver Tester: | Tortoise |
| --- | --- |
| Test Date: | 2007/01/16 |
| Comments\Attachments: |  |